Synopsis of Major Project

Date: 12/03/2026

Major Project Title: AURA - Autonomous User Realization & Assembly

Project Supervisor: Ms. Indu Sharma (Assistant Professor)
Project Coordinator: Dr. Naveen Sharma (Coordinator, CSE)

| Program: B.Tech (CSE) |  | Year/Semester: 8th Semester |  |
| --- | --- | --- | --- |
| S. No. | Enrolment No. | Name | Signature |
| 1 | 12196502722 | SANTOSH KUMAR |  |

Major Project Summary :

AURA - Autonomous User Realization & Assembly

AURA is a complete prototype-level cyber-physical electronics assistance system that unifies physical component localization, quantity-aware inventory management, and deterministic AI-assisted circuit composition. The platform is intended for students, makers, and small laboratories that need to design circuits from the components they actually own rather than from idealized virtual inventories.

The system combines an ESP32-based host with local search capability, optional smartphone interaction over Bluetooth, and compact low-power distributed nodes built around ATtiny404 and nRF24L01+ devices with WS2812B visual indication and optional buzzer cues. One node can cover a dense row of nearby drawers or bins, while multiple nodes can be deployed across a room for distributed storage layouts.

Key capabilities include addressed component lookup, value-specific and quantity-aware stock tracking, manual and voice-assisted search, deterministic JSON circuit representation, local rule checking before AI assistance, structured proposal review, and simultaneous highlighting of all components required for a selected build.

On the software side, AURA maintains canonical JSON as the source of truth for circuit state and inventory-linked recommendations. Local deterministic calculations and supported-pattern checks run before open-ended AI reasoning, and preview behavior is presented clearly as illustrative rather than electrically verified.

On the inventory side, the system records component type, value, quantity, physical address, and optional project metadata so it can answer not only where a component is stored but also whether sufficient quantity exists for a planned circuit. This is especially valuable for common cases such as resistor assortments, where existence and usable count are different questions.

The present work therefore demonstrates AURA as an end-to-end prototype workflow: search or request a circuit, filter alternatives against owned inventory, inspect a deterministic circuit representation, and trigger distributed physical highlighting for the required parts. PCB miniaturization, deployment hardening, and broader companion-app polish remain future engineering refinements rather than unresolved architectural gaps.

Objectives:

The primary objectives of the AURA project are:

• To develop an integrated cyber-physical workflow that links physical component retrieval with digital circuit planning.

• To design an ESP32-based host unit with local search UI, metadata storage, Bluetooth smartphone bridging, and nRF24L01+ coordination.

• To build compact low-power distributed locator nodes using ATtiny404, nRF24L01+, WS2812B, and optional buzzer cues.

• To support scalable addressing so one node can manage dense component rows while multiple nodes can cover larger rooms and separated storage zones.

• To maintain value-aware, quantity-aware, and location-aware inventory records for exact component variants.

• To support component lookup through manual search, smartphone-assisted interaction, and future voice-assistant style invocation.

• To implement a deterministic JSON-first circuit workspace in which explicit structured circuit data remains the source of truth.

• To combine local calculations and rule-based checks with AI assistance so recommendations remain constrained, inspectable, and reviewable.

• To support structured compare, revise, and apply workflows instead of uncontrolled free-form AI-only output.

• To enable whole-project highlighting so all required parts for an accepted build can be located together at once.

• To create reusable component metadata for pin mapping, constraints, media references, and project linkage.

• To provide learner-friendly interfaces that explain circuits, inventory status, and physical retrieval steps.

• To demonstrate the system as a complete prototype while reserving PCB refinement, packaging, and larger-scale validation for future work.

Research Paper Topic:

The research and technical background for AURA draws from the following interconnected domains:

1. Cyber-Physical Electronics Assistance

- Linking digital circuit planning with real-world component retrieval

- Inventory-constrained prototyping workflows

- Addressed physical location mapping across distributed storage

- Host-mediated coordination between smartphone, controller, and nodes

2. Deterministic Circuit Representation and Review

- Canonical JSON as the source of truth for circuit state

- Local calculations, rule checks, and supported-pattern analysis

- Structured patch and proposal review workflows

- Clear separation between illustrative preview and verified behavior

- 3. Inventory and Metadata Intelligence

Value-aware and quantity-aware component records

- Physical addresses, drawer mapping, and project-linked tagging

- Pin metadata, package constraints, and reusable component descriptors

- Stock sufficiency checks for exact component variants

- Optional notes, images, and lookup aids for retrieval

- 4. Low-Power Distributed Node Architecture

ESP32 host coordination with local UI and Bluetooth bridging

- ATtiny404-based compact node control

- nRF24L01+ based addressed wireless communication

- WS2812B-based visual indication for individual or grouped components

- Optional buzzer and project-marker modes

5. Human-Centered Interaction

- Smartphone-assisted search and inventory browsing

- Voice-assisted component lookup through external assistant integration

- Guided circuit explanation for learners and makers

- Whole-project visual retrieval before assembly

6. AI-Assisted but Constrained Design Workflows

- Structured AI proposals grounded in deterministic circuit data

- Inventory-aware circuit recommendation and alternative generation

- Hybrid AI plus rule-engine assistance

7. Educational and Laboratory Use Cases

- Student lab assistance and guided prototyping

- Hobbyist workspace organization

- Small laboratory and maker-space inventory coordination

8. Deployment and Scaling

- PCB refinement and node miniaturization

- Large-room multi-node deployment

- Reliability, packaging, and deployment hardening

Base Paper / Technical Reference Links:

AURA sits at the intersection of embedded systems, wireless locator networks, inventory-aware design workflows, and constrained AI assistance. The following references form the technical basis of the project:

1. ESP32 Technical Reference Manual

Espressif Systems; Topic: host control, connectivity, Bluetooth bridging, and embedded user interaction; Link: https://www.espressif.com/

2. nRF24L01+ Product Specification

Nordic Semiconductor; Topic: addressed low-power wireless communication for distributed locator nodes; Link: https://www.nordicsemi.com/

3. tinyAVR 0-Series Family Data Sheet

Microchip Technology; Topic: compact low-power node controller design using ATtiny404-class devices; Link: https://www.microchip.com/

4. WS2812B Intelligent Control LED Documentation

Topic: individually addressable visual indication for component and project highlighting.

5. Structured Output and Tool-Augmented Language Models

Toolformer: Language Models Can Teach Themselves to Use Tools; Relevance: AI assistance grounded by tools, rules, and structured outputs; Link: https://arxiv.org/abs/2302.04761

6. Human-Centered AI and Interaction Design

Topic: trustworthy, reviewable, and human-centered AI assistance in engineering and education.

7. Practical Electronics Reference

The Art of Electronics; Relevance: component behavior, circuit reasoning, and applied prototyping practice.

Resource Requirement:

The resource requirements for the current AURA prototype are summarized in the following table.

| RESOURCE TYPE | CURRENT REQUIREMENT |
| --- | --- |
| Hardware | Development laptop or PC for software, firmware, and documentation work. ESP32-based host prototype for local search, control, and Bluetooth bridging. ATtiny404 + nRF24L01+ node prototypes with WS2812B indication and optional buzzer cues. Breadboard, wiring, power supplies, soldering support, and PCB prototyping materials. |
| Software | Node.js-based development environment for the workspace and backend services. Frontend tooling, testing utilities, and version-control support. Firmware toolchains for ESP32 and tinyAVR-based node development. Documentation, diagramming, and presentation tools for academic submission. |
| Data / Design Assets | Inventory records with component type, value, quantity, and physical storage address. Canonical JSON circuit models and sample inventory-aware circuit configurations. Prototype photographs, UI screenshots, datasheets, and technical references. |
| Human Resources | Student researcher and developer. Project supervisor and project coordinator for academic review. Peer feedback for usability checks and workflow validation. |
| Other Resources | Internet access for literature review, package management, and optional AI-service integration. Basic lab tools for assembly, testing, and physical storage setup. |

Schedule of Major Project Work Along with Research Paper:

| MONTH | ACTIVITIES | DETAILS |
| --- | --- | --- |
| Month 1 (January) | Problem Definition and Architecture Planning | • Finalize the project scope, title, and major objectives. • Review literature related to cyber-physical electronics assistance, distributed nodes, and deterministic circuit workflows. • Define the AURA architecture covering host, nodes, inventory management, and the circuit workspace. |
| Month 2 (February) | Prototype Integration and Core Implementation | • Build and integrate the ESP32 host path and the low-power ATtiny404 + nRF24L01+ locator nodes. • Implement WS2812B-based visual indication, component addressing, and inventory-linked lookup. • Develop the deterministic workspace, local rule-checking flow, and AI-assisted proposal path. |
| Month 3 (March) | Validation, Documentation, and Academic Submission | • Validate component lookup, quantity awareness, multi-part highlighting, and circuit-to-retrieval workflow. • Capture prototype photographs, UI screenshots, and technical figures for synopsis, report, and conference paper. • Prepare final documentation, presentation flow, and review material for academic submission. |

Signature of Project Coordinator:

Approval by Academic Project Committee (Official)

| Member | Signature | Remark (Approved/Not Approved) |
| --- | --- | --- |